Safety Performance Evaluation of Reinforcement Learning Algorithms with Transformer-based Policies in High-risk Autonomous Driving

Jun Xuan Ng*

Abstract—This manuscript provides a systematic literature review on the safety performance of Transformer-based Deep Reinforcement Learning (DRL) algorithms in autonomous driving scenarios. While reinforcement learning has shown potential in autonomous driving tasks, the differences in safety performance among different reinforcement learning algorithms using the Transformer architecture remain unclear. Through a systematic analysis, this study finds that although Soft Actor-Critic (SAC) algorithms generally offer higher safety in complex tasks, no single algorithm possesses an absolute advantage in all scenarios. Furthermore, this study establishes a risk hierarchy through transitive reasoning and identifies turn intersections as the most challenging scenario.

Keywords—Autonomous Driving, Deep Reinforcement Learning, Transformer, Safety Performance, Systematic Literature Review

# Introduction

## Background of the Research

With the advent of the era of artificial intelligence and automation, many tasks no longer require human intervention (Frey & Osborne, 2017). Autonomous driving is one such trend, with most car manufacturers integrating Level 4+ autonomous driving capabilities (Huynh & Nguyen, 2024, p. 1). However, despite rapid development, autonomous driving still faces many challenges. (Dosovitskiy et al., 2017, p. 7) indicate that rule-based control methods are vulnerable in dynamic and complex environments, and safety significantly decreases when sensors are limited or the environment is highly variable. Furthermore, the use of deep learning relies on large amounts of labeled data, which requires significant human effort and time, resulting in high costs.

## Problem Statement

Autonomous driving still faces many safety challenges in complex driving scenarios (Luo et al., 2025, p. 1). Although reinforcement learning has shown potential in autonomous driving tasks, the differences in safety performance among different RL algorithms using the Transformer architecture remain unclear. Therefore, it is difficult to determine which reinforcement learning algorithm is most suitable for developing and designing autonomous driving models.

## Research Purpose

The purpose of this review is to comprehensively analyze and synthesize research on the safety of Transformer-based reinforcement learning algorithms in high-risk autonomous driving scenarios. Specifically, this review purpose is to:

- Classify commonly used Transformer architectures in autonomous driving tasks.

- Compare the safety performance of these algorithms (PPO, SAC, DDPG/TD3).

- Rank the risks of various scenarios based on literature data.

## Objectives of the Research

- To systematically identify and categorize the most commonly used Transformer-based model and reinforcement learning algorithms in autonomous driving tasks.

- To quantitatively compare the safety performance of reinforcement algorithms under comparable driving scenarios.

- To rank the risk levels of different high-risk autonomous driving scenarios using a consistent set of safety metrics.

## Research Questions

The main research question is under a Transformer-based policy network architecture, what are the differences in safety performance among different reinforcement learning algorithms in high-risk autonomous driving scenarios?

To answer this question, this study explores the following sub-questions:

- What are some commonly used Transformer-based models in high-risk autonomous driving tasks?

- What are the differences in safety performance among different reinforcement learning algorithms in similar high-risk driving scenarios?

- What are the differences between different high-risk scenarios?

## Project Scope

This study aims to systematically evaluate the safety performance of different reinforcement learning algorithms under the Transformer architecture in high-risk autonomous driving scenarios.

### Technical Scope

This study focuses on model architecture, algorithm performance, scenario risk, and safety metrics.

- This study uses Transformer-based models as the core component of the state encoder or policy network.

- This study primarily compares the safety performance differences of PPO, SAC, and DDPG/TD3 under the same Transformer architecture.

- This study will focus on the most common high-risk scenarios identified in the literature, including intersections, ramp merging, roundabouts, and highways.

- This study mainly uses quantitative safety metrics such as collision rate and success rate for evaluation.

### Data Sources and Screening Scope

This literature review only includes the top 100 papers in Google Scholar and all papers published in the IEEE Xplore database between 2019 and 2025.

Only papers containing the Transformer architecture, specific reinforcement learning algorithms (PPO, SCA, DDPG/TD3), and safety-related metrics are considered. Papers involving non-autonomous driving scenarios, such as drones and driver state detection, will not be considered. Review articles and articles that are unavailable are not within the scope of this literature review.

### Output and Contribution Scope

- Compare the safety performance of various algorithms under the same transformer model

- Rank the driving scenarios of each paper by risk level.

- Conclude the common types of transformers used in autonomous driving tasks.

## Significance of the Research

This study, through a systematic literature review and quantitative comparative analysis, explores the safety performance of different reinforcement learning algorithms under the Transformer architecture in high-risk autonomous driving scenarios. It has the following important theoretical and practical implications:

- This study constructs a comparative framework by collecting and extracting data from published papers, which helps to more objectively evaluate the differences in algorithm performance.

- This study also collects and extracts the performance of each algorithm in different scenarios to define and rank the risk levels of these scenarios.

- This study also reviews all types of Transformer-based models used and provides architectural options for future research.

These results will provide a reference for autonomous driving system developers in algorithm selection and optimization.

# Literature review

## Project Scope

Traditional rule-based methods require manually pre-setting driving strategies, which can lead to safety issues as scenarios become more complex (G. Zhao et al., 2025, p. 1). This chapter reviews the Transformer architecture and specifies reinforcement learning algorithms, explaining why the Transformer is suitable for application in the field of autonomous driving. Furthermore, this chapter provides definitions for various driving scenarios.

## Systematic Literature Review Methodology

### Search Strategies

- Database: Google Scholar, IEEE Xplore

- Years: 2019-2025

- Keyword: ("Transformer") AND ("Reinforcement Learning" OR "Deep Reinforcement Learning" OR DRL OR RL OR "Q-learning" OR "Policy Gradient") AND ("Autonomous Driving" OR "Autonomous Vehicle" OR "Self-driving" OR "Intelligent Vehicle" OR CAV) AND ("Collision Avoidance" OR "Time to Collision" OR TTC OR Safety OR "Near-miss" OR Accident OR Crash OR "Risk Assessment")

### Include criteria

Included papers must meet all of the following criteria:

- The research must use a Transformer-based model as its core model or state encoder.

- Papers must cover at least one specified reinforcement learning algorithm, including PPO, SAC, or DDPG/TD3.

- Quantifiable safety assessment data must be provided.

### Exclude criteria

Included papers cannot contain any of the following:

- All review articles, surveys, tutorials, or systematic reviews are excluded because they do not provide original experimental data.

- Research on non-autonomous driving, such as driver fatigue detection, is excluded. Research applied to off-road traffic scenarios (e.g., off-road vehicles) is also excluded.

### Screening process

##### Figure 2.1 Screening Flow chart.

## Specific Applications of Transformer-RL in Autonomous Driving

In recent years, the Transformer architecture, with its self-attention mechanism, has demonstrated outstanding capabilities in handling long-range dependencies and complex relationships in modeling data, thus revolutionizing the field of autonomous driving (Doroudian & Taghavifar, 2024, p. 1). Transformer applications in reinforcement learning involve two embedding methods: one is using the Transformer as a state encoder, and the other is directly using the Transformer as a policy/actor. These two application methods will be detailed below.

### State Encoder Design (Multimodal Transformer)

##### Figure 2.2 Transformer as a state encoder.

In driving tasks, multilayer perceptrons (MLPs) in reinforcement learning cannot directly process multimodal data; however, autonomous driving tasks rely on data from multiple sensors, such as cameras and LiDAR point clouds.

The Transformer architecture overcomes these limitations through its self-attention mechanism (Vaswani et al., 2023). This mechanism computes relationships between sensor data in different formats, enabling the Transformer to find features in one modality (e.g., visual objects) and relevant features in another modality (e.g., vehicle states such as steering and linear velocity). For example, the DAD-RL framework proposed by (Chowdhury et al., 2024) employs a multimodal input design, with input data including historical vehicle trajectories and bird's-eye view (BEV) contextual information. The historical trajectory contains the position, heading, speed, and lane information of surrounding vehicles over the past 2.1 seconds, while the BEV image modality contains a drivable area map and waypoint maps.

Furthermore, a challenge faced by multilayer perceptrons (MLPs) is that driving decisions in autonomous driving tasks inherently depend on both the current and historical states, and MLPs lack the ability to process time-series data. While models capable of handling time-series data, such as recurrent neural networks (RNNs) and long short-term memory networks (LSTMs), can be introduced as state encoders, their performance remains poor when dealing with excessively long time-series data. In contrast, the Transformer, utilizing a parallelizable self-attention mechanism, can efficiently process data with large time spans (Fan et al., 2024).

### Policy Network Architecture (Transformer as Policy/Actor)

##### Figure 2.3 Transformer as a Policy/Actor.

Compared to MLPs, Transformers are better suited as policy networks. MLPs lack explicit mechanisms for modeling temporal dependencies. Transformers, however, can analyze relationships between data spanning long time periods through self-attention mechanisms.

In the paper (Doroudian & Taghavifar, 2024), the authors incorporated Transformers as part of a policy network and combined it with the PPO algorithm. The PPO Transformer (TPPO) achieved a 98% success rate and only a 2% collision rate, while the non-Transformer PPO achieved a 42% success rate and a 12% collision rate. The safety of PPO with Transformers is significantly improved compared to the non-Transformer PPO.

## Comparative Applications of RL Algorithms in Autonomous Driving Decision-Making

### Application Research of PPO

In traditional reinforcement learning frameworks, policy gradient methods are highly sensitive to the learning rate. A learning rate that is too slow leads to slow parameter updates, while a learning rate that is too high can have disastrous consequences. Since the agent's behavior directly alters its future state, excessively large policy updates can cause the agent to explore discontinuous data, affecting data quality and ultimately leading to policy collapse.

To overcome this problem, (Schulman, Levine, et al., 2017) proposed Trust Domain Policy Optimization (TRPO). The core of TRPO lies in using the Kullback-Leibler divergence to limit the difference between the old and new policies. However, TRPO incurs extremely high computational costs. Therefore, (Schulman, Wolski, et al., 2017) proposed Proximal Policy Optimization (PPO). PPO introduces a "Clipped Surrogate Objective", which restricts the probability ratio of the old and new policies to a predefined range. This mechanism not only significantly lowers the implementation threshold of the algorithm but also greatly improves sample efficiency. Experiments show that PPO exhibits excellent performance in both continuous control and discrete action space tasks (Schulman, Wolski et al., 2017).

### Application Research of SAC

Soft Actor-Critic (SAC) is an offline deep reinforcement learning algorithm based on maximum entropy reinforcement learning and the stochastic actor-critic algorithm. It is particularly suitable for tasks with continuous action spaces, offering higher sample utilization and more stable training results (Haarnoja et al., 2018). SAC is a two-tube neural network (Haarnoja et al., 2018).

SAC uses one actor and two Q-function networks and can also be considered as critics. The actor's output is the parameters of the probability distribution of actions (usually the mean and standard deviation), for example, the throttle is μ=0.5，σ=0.28. A small amount of noise is added to the standard deviation part to help the actor explore better actions. The critic part is similar to TD3, they both use two independent Q-function networks.

### Application Research of DDPG

Reinforcement learning methods for discrete action spaces, such as DQN, cannot be directly applied to continuous action spaces because the "maximization" operation of continuous actions requires complex optimization at each step, resulting in high computational cost and instability. DDPG combines the ideas of Deterministic Policy Gradient (DPG) and Deep Q-Networks (DQN) to propose an off-policy, model-agnostic actor-critic algorithm suitable for continuous action spaces (Lillicrap et al., 2019).

DDPG is based on the DPG framework, where:

- Actor: Outputs deterministic continuous actions.

- Critic: Evaluates the Q-values of state-action pairs.

By using a replay buffer and target networks, DDPG borrows the stable training mechanism of DQN, achieving efficient learning in continuous action spaces (Lillicrap et al., 2019).

### Application Research of TD3

A problem exists in the DQN algorithm: Q-value overestimation. Since DDPG inherits from DQN, it also inherits this problem. This issue arises because the DQN critic overestimates the Q-value when evaluating actions due to noise, and actors tend to learn those overestimated actions, ultimately leading the policy to suboptimal solutions or even training failure.

To address this problem, TD3 was proposed. This algorithm introduces three main features:

- Delayed actor updates: This allows the critic to stabilize before the actor learns actions closer to the actual Q-value.

- Adding two independent critics: This ensures that the smallest Q-value is selected. While this might still lead the actor to learn actually underestimated actions, (Fujimoto et al., 2018) show that learning underestimated actions is safer than learning overestimated ones.

- Target policy smoothing: Adding a small amount of noise to the target action smooths the Q-value estimation and improves robustness.

## Definition and Research of High-Risk Autonomous Driving Scenarios

### Classification and Definition of High-Risk Scenarios

To cultivate robust decision-making in autonomous agents, exposure to high-risk scenarios during training is imperative. Analogous to the adage that calm seas do not produce skilled sailors, a benign training environment fails to equip agents with the necessary skills to handle unexpected real-world situations.

- Merging: A scenario where vehicles, without traffic signal guidance, need to observe, predict, and negotiate right-of-way with traffic flows from multiple directions.

- Roundabout: A multi-lane roundabout where vehicles need to merge, cycle, and exit within a continuous traffic flow.

- Turn intersection: A turn intersection includes: unsignaled intersections, four-way intersections, left turns, and unprotected left turns. I categorize them all as turn intersections. It is a scenario where vehicles, without traffic signals, must observe, predict, and negotiate right-of-way with multi-directional traffic flows (including oncoming vehicles, pedestrians, and non-motorized vehicles).

- Highway: A high-speed, multi-lane environment. In some papers, the highway scenario may include overtake tasks.

## Risk Assessment Index System

Each paper uses different metrics to measure the performance of its model and algorithm. Below is a summary of the most commonly used metrics across these papers. While some papers may use the same metrics, their measurement methods may vary slightly.

- Collision Rate: This is the most frequently used safety metric after data extraction. It calculates the proportion of collisions that occur in a test scenario.

- Collisions Time: This refers to the total number of collisions that occur when the autonomous vehicle completes a navigation task in each test scenario.

- Success Rate: This is the second most commonly used metric. This metric includes not only the collision-free task completion rate but may also include other factors, such as reaching the destination within the specified time.

- Lane Departure Rate: Commonly used to evaluate whether a vehicle can stably maintain its position near the center of the lane, avoiding excessive swaying or deviation. This metric is usually measured in centimeters (cm).

# Algorithm performance comparison

## Introduction

This chapter aims to compare the safety performance of different deep reinforcement learning (DRL) algorithms in autonomous driving decision-making tasks through a systematic analysis of experimental data from existing literature. Although algorithms such as PPO, SAC, and DDPG/TD3 each have their own theoretical focus, their actual safety and effectiveness under the complex Transformer architecture still need to be verified through quantitative metrics. This chapter will focus on the horizontal and vertical performance evaluation of various mainstream algorithms based on key indicators such as collision rate, collision time, and success rate.

## Comparative Analysis of PPO and SAC

#### Table 3.1 Performance Comparison between PPO and SAC

| Paper title | PPO Performance | SAC Performance | Conclusion (Low performance < High performance) |
| --- | --- | --- | --- |
| Augmenting Reinforcement Learning With Transformer-Based Scene Representation Learning for Decision-Making of Autonomous Driving (Liu et al., 2024) | Collision rate: Min: 34%. Mean: 51.2%. Max: 64%. | Collision rate: Min: 22%. Mean: 35%. Max: 52%. | PPO < SAC |
| DAD-RL for Autonomous Decision-Making in Dynamic Environment (Chowdhury et al., 2024) | Collision rate: Min: 34%. Mean: 51.2%. Max: 64%. | Collision rate: Min: 22%. Mean: 35%. Max: 54%. | PPO < SAC |
| Efficient Maximum Entropy Reinforcement Learning Framework Guided by Expert Policies: An Application in Autonomous Driving (L. Gao et al., 2025) | Success rate: Min: 36%. Mean: 37%. Max: 38%. | Success rate: Min: 65%. Mean: 68.5%. Max: 72%. | PPO < SAC |

#### Table 3.1 (continued)

| Paper title | PPO Performance | SAC Performance | Conclusion (Low performance < High performance) |
| --- | --- | --- | --- |
| Improving Reinforcement Learning with Expert Demonstrations and Vision Transformers for Autonomous Vehicle Control (Elallid et al., 2024) | Collision rate: 20%. Success rate: 80%. | Collision rate: 6%. Success rate: 94%. | PPO < SAC |
| SoftCTRL: Soft conservative KL-control of Transformer Reinforcement Learning for Autonomous Driving (Huynh & Nguyen, 2024) | Collision time: Mean: 20.87 ±1.14. | Collision time: Mean: 1.87 ±0.25. | PPO < SAC |

Table 3.1 shows all the performance data that the papers include both PPO and SAC algorithms. In the data from these five papers, the SAC algorithm is significantly more secure than the PPO algorithm.

(Liu et al., 2024), PPO has a minimum collision rate of 34%, an average of 51.2%, and a maximum collision rate of 64%, while SAC has a minimum collision rate of 22%, an average of 35%, and a maximum collision rate of 52%. This shows that SAC's collision rate is lower than PPO's collision rates. It is noteworthy in this table that the data from (Liu et al., 2024) and (Chowdhury et al., 2024) are very similar. This is because (Chowdhury et al., 2024) explicitly built upon (Liu et al., 2024)’s framework, using it as a baseline for comparison. This represents standard academic practice where new methods are evaluated against established benchmarks.

Regarding success rate, (L. Gao et al., 2025) shows that PPO's success rate is also lower than SAC's in terms of minimum, average, and maximum. (Elallid et al., 2024) used two metrics: in terms of collision rate, PPO (20%) was significantly higher than SAC (4%), while in terms of success rate, PPO (80%) and SAC (94%), both metrics showed that PPO was inferior to SAC. (Huynh & Nguyen, 2024) used a rarer metric: in terms of the number of collisions, PPO was nearly 11 times that of SAC.

## Comparative Analysis of SAC and DDPG/TD3

#### Table 3.2 Performance Comparison between SAC and DDPG/TD3

| Paper title | SAC Performance | DDPG/TD3 Performance | Conclusion (Low performance < High performance) |
| --- | --- | --- | --- |
| Improving policy training for autonomous driving through randomized ensembled double Q-learning with Transformer encoder feature evaluation (Fan et al., 2024) | Success rate: Min: 16.2%. Max: 16.4%. | Success rate: Min: 16.3%. Max: 16.4%. | SAC ≈ DDPG/TD3 |

Table 3.2 shows performance data for both the SAC and TD3 (DDPG) algorithms. In (Fan et al., 2024), the performance of SAC and TD3 is almost identical.

## Comparative Analysis of Merging and Intersection Scenarios DDPG/TD3 and PPO

#### Table 3.3 Performance Comparison between DDPG/TD3 and PPO.

| Paper title | DDPG/TD3 Performance | PPO Performance | Conclusion (Low performance < High performance) |
| --- | --- | --- | --- |
| Autonomous Intersection Management via Prior-Enhanced Multi-Agent Constrained Decision Transformer (R. Zhao et al., 2025) | Collision rate: Min: 44.37%. Mean: 58.98%. Max: 69.86%. | Collision rate: Min: 60.25%. Mean: 66.95%. Max: 77.99%. | PPO < DDPG/TD3 |
| Constraint-Guided Behavior Transformer for Centralized Coordination of Connected and Automated Vehicles at Intersections (Zhao et al., 2024) | Collision rate: Min: 66.73%. Mean: 68.295%. Max: 69.86%. | Collision rate: Min: 64.265. Mean: 71.125%. Max: 77.99%. | PPO < DDPG/TD3 |

In (R. Zhao et al., 2025; Zhao et al., 2024), the average collision rates of DDPG/TD3 were 58.98% and 68.295%, respectively, while those of PPO were 66.95% and 71.125%. These results indicate that PPO performs worse than DDPG/TD3.

## Comparative Analysis of Merging and Intersection Scenarios PPO, SAC and DDPG/TD3

#### Table 3.4 Performance Comparison between PPO, SAC and DDPG/TD3

| Paper title | PPO Performance | SAC Performance | DDPG/TD3 Performance | Conclusion (Low performance < High performance) |
| --- | --- | --- | --- | --- |
| Safety-constrained Reinforcement Learning with Interaction-aware for Decision-making of Autonomous Driving (Zhang et al., 2025) | Success rate: Min: 42%. Mean: 51.33%. Max: 58%. | Success rate: Min: 33%. Mean: 38.7%. Max: 48%. | Success rate: Min: 7%. Mean: 18.7%. Max: 37%. | DDPG/TD3 < SAC SAC < PPO DDPG/TD3 < PPO |

(Zhang et al., 2025) is the most direct in determining which algorithm is optimal because it directly compares the three. (Zhang et al., 2025) shows that PPO is the best, with an average success rate of 51.33%, followed by SAC with an average success rate of 38.7%, and finally DDPG/TD3 with an average success rate of 18.7%.

## Analysis summary

#### Table 3.5 Comparative Performance Analysis of Reinforcement Learning Algorithms.

| No | Conclusion (Low performance < High performance) | Support paper |
| --- | --- | --- |
| 1 | PPO<SAC | (Liu et al., 2024) (Chowdhury et al., 2024) (L. Gao et al., 2025) (Elallid et al., 2024) (Huynh & Nguyen, 2024) |
| 2 | SAC≈DDPG/TD3 | (Fan et al., 2024) |
| 3 | PPO<DDPG/TD3 | (R. Zhao et al., 2025) (Zhao et al., 2024) |
| 4 | DDPG/TD3<SAC | (Zhang et al., 2025) |
| 5 | SAC<PPO | (Zhang et al., 2025) |
| 6 | DDPG/TD3<PPO | (Zhang et al., 2025) |

Table 3.5 summarizes the performance comparisons of the PPO, SAC, and DDPG/TD3 algorithms in the main studies. This table reveals two contradictions. The first is the comparison between PPO and SAC. While most studies (5 supporting papers) agree that PPO < SAC, indicating that SAC has higher efficiency in autonomous driving tasks, a few studies (1 paper) reach the opposite conclusion: SAC < PPO. The second is the comparison between PPO and DDPG/TD3. Two papers support the claim that PPO < DDPG/TD3, while only one paper supports PPO > DDPG/TD3.

In conclusion, there are a total of 8 papers covering the PPO algorithm, 7 papers covering the SAC algorithm, and 3 papers covering the DDPG/TD3 algorithm.

This study, through a systematic literature review, finds that PPO, SAC, and DDPG/TD3 do not have an absolute superiority or inferiority in autonomous driving decision-making tasks. Their performance is highly dependent on task attributes, state representation complexity, reward function design, and environmental stochasticity. For example, PPO performs better in safety-constrained tasks requiring stable policy updates, while SAC is more advantageous in scene representation learning where exploration efficiency is crucial. Therefore, algorithm selection should be based on a thorough analysis of task characteristics.

# High-risk scenario assessment

## Background of the Research

(Lu et al., 2025, p. 1) points out that autonomous driving tasks have several characteristics, including high vehicle density, diverse traffic participants, and complex traffic rules. These factors will affect the safety and robustness of autonomous driving algorithms. This chapter summarizes and extracts data from all the papers and uses this data to rank the risk levels of various scenarios. This chapter will use logical deduction to construct a comprehensive ranking of scenario risk levels.

## Comparative Analysis of Merging and Intersection Scenarios

## Table 4.1 Risk Assessment between Merging and Intersection

| Paper title | Merging risk assessment | Intersection risk (Left turn) assessment | Conclusion (Low risk < High risk) |
| --- | --- | --- | --- |
| Efficient Maximum Entropy Reinforcement Learning Framework Guided by Expert Policies: An Application in Autonomous Driving (L. Gao et al., 2025) | Success rate: Min: 38%. Mean: 69.14%. Max: 92%. | Success rate: Min: 36%. Mean: 72%. Max: 96%. | Intersection (Left turn) < Merging |

Table 4.1 compares the performance of autonomous driving in two high-risk scenarios—merging and intersections. As you can see, the average success rate at intersections is 72%, and at merging it's 69.14%, with little difference between the two. This indicates that in this study, the risks of these two scenarios are comparable, but merging is slightly more dangerous.

## Comparative Analysis of Roundabout and Intersection

#### Table 4.2 Risk Assessment between Roundabout and Right turn Intersection

| Paper title | Roundabout risk assessment | Intersection (Right turn) risk assessment | Conclusion (Low risk < High risk) |
| --- | --- | --- | --- |
| Improving Reinforcement Learning with Expert Demonstrations and Vision Transformers for Autonomous Vehicle Control (Elallid et al., 2024) | Collision rate: 6% | Collision rate: 32% | Roundabout < Intersection (Right turn) |

#### Table 4.3 Risk Assessment between Roundabout and Intersection

| Paper title | Roundabout risk assessment | Intersection risk assessment | Conclusion (Low risk < High risk) |
| --- | --- | --- | --- |
| Autonomous Driving Agents for Safe Road Junctions Through Deep Reinforcement Learning (Yun & Cao, 2024) | Collision rate: Min: 7.2%. Mean: 14.12%. Max: 20.47%. | Collision rate: Min: 56.88%. Mean: 62.28%. Max: 65.51%. | Roundabout < Intersection |

Table 4.2 and 4.3 compares the performance of autonomous driving models in roundabout and intersection scenarios. (Elallid et al., 2024) shows that autonomous driving models are more prone to accidents in intersection scenarios, with a collision rate of 32% compared to 6% in roundabout scenarios. Beside, (Yun & Cao, 2024) also indicates that the intersection scenario is higher risk than roundabout.

## Comparative Analysis of Intersection and Highway

#### Table 4.4 Risk Assessment between Intersection and Highway

| Paper title | Intersection risk assessment | Highway risk assessment | Conclusion (Low risk < High risk) |
| --- | --- | --- | --- |
| Empirical Evaluation of Deep Learning based Autonomous Driving System with Real-Time Scene Classification Procedure (Prasath A & Navaz K, 2025) | Lane deviation error: Min: 6.47cm. Mean: 9.77cm. Max: 12.4cm. | Lane deviation error: Min: 6.22cm. Mean: 8.96cm. Max: 13.45cm. | Highway < Intersection |
|  | Intersection risk (Left turn) assessment | Highway risk assessment | Conclusion (Low risk < High risk) |
| Pre-trained Transformer-Enabled Strategies with Human-Guided Fine-Tuning for End-to-end Navigation of Autonomous Vehicles (Hu et al., 2024) | Collision rate: Min: 0%. Mean: 21.93%. Max: 58.7%. | Collision rate: Min: 0%. Mean: 8.84%. Max: 38.7%. | Highway < Intersection (Left turn) |

Table 4.4 contains two papers, each using a different metric: lane deviation error and collision rate. Both papers show that the risk is higher in intersection scenarios than in highway scenarios. In (Prasath A & Navaz K, 2025), the average lane deviation error for intersections is 9.77 cm, while the average lane deviation error for highways is 8.96 cm. (Hu et al., 2024) shows an average collision rate of 21.93% for intersections, compared to 8.84% for highway scenarios.

## Comparative Analysis of Merging, Roundabout and Intersection

#### Table 4.5 Risk Assessment between Roundabout and Intersection

| Paper title | Merging risk assessment | Roundabout risk assessment | Intersection (Left turn) risk assessment | Conclusion (Low risk < High risk) |
| --- | --- | --- | --- | --- |
| Augmenting Reinforcement Learning With Transformer-Based Scene Representation Learning for Decision-Making of Autonomous Driving (H. Liu et al., 2024) | Collision rate: Min: 4%. Mean: 39%. Max: 100%. | Collision rate: Min: 16%. Mean: 40.56%. Max: 100%. | Collision rate: Min: 12%. Mean: 32.66%. Max: 54%. | Merging < Roundabout Intersection (Left turn) < Roundabout Intersection (Left turn) < Merging |
| Deep Attention Driven Reinforcement Learning (DAD-RL) for Autonomous Decision-Making in Dynamic Environment (Chowdhury et al., 2024) | Collision rate: Min: 2%. Mean:31.25%. Max:100%. | Collision rate: Min:4%. Mean:34.17%. Max:100%. | Collision rate: Min:0%. Mean:25%. Max:54%. | Merging < Roundabout Intersection (Left turn) < Roundabout Intersection (Left turn) < Merging |

Table 4.5 contains two papers that use the same metric and include scenarios involving merging, roundabout, and intersection. (H. Liu et al., 2024)'s average collision rates are 39% for merging, 40.56% for roundabouts, and 32.66% for intersections. (Chowdhury et al., 2024)’s average collision rates are 31.25% for merging, 34.17% for roundabout, and 25% for intersection. Both papers show that roundabouts are the most dangerous, followed by merging, and lastly, intersections.

## Comparative Analysis of Merging, Intersection and Highway

#### Table 4.6 Risk Assessment between Merging, Intersection and Highway

| Paper title | Merging risk assessment | Intersection (Left turn) risk assessment | Highway risk assessment | Conclusion (Low risk < High risk) |
| --- | --- | --- | --- | --- |
| Safety-constrained Reinforcement Learning with Interaction-aware for Decision-making of Autonomous Driving (Zhang et al., 2025) | Success rate: Min: 7%. Mean: 44%. Max: 94%. | Success rate: Min: 12%. Mean: 49.5%. Max: 97%. | Success rate: Min: 37%. Mean: 60.75%. Max: 100%. | Intersection (Left turn) < Merging Highway < Intersection (Left turn) Highway < Merging |

(Zhang et al., 2025) includes scenarios involving merging, intersections, and highways. The average success rate of the first paper was 44% for merging, 49.5% for intersection, and 60.75% for highway. This paper shows that merging is the most dangerous, followed by intersections, and lastly highways.

## Analysis summary

#### Table 4.7 Summary of Scenario Risk Hierarchies from Primary Studies

| No | Conclusion (low risk < high risk) | Support paper |
| --- | --- | --- |
| 1 | Intersection(Left turn) < Merging | (L. Gao et al., 2025) (H. Liu et al., 2024) (Chowdhury et al., 2024) (Zhang et al., 2025) |
| 2 | Roundabout < Intersection (Right turn) | (Elallid et al., 2024) |
| 3 | Roundabout < Intersection | (Yun & Cao, 2024) |
| 4 | Intersection (Left turn) < Roundabout | (H. Liu et al., 2024) (Chowdhury et al., 2024) |
| 5 | Highway < Intersection | (Prasath A & Navaz K, 2025) |
| 6 | Highway < Intersection (Left turn) | (Hu et al., 2024) (Zhang et al., 2025) |
| 7 | Merging < Roundabout | (H. Liu et al., 2024) (Chowdhury et al., 2024) |
| 8 | Highway < Merging | (Zhang et al., 2025) |

Table 4.7 summarizes the risk comparison findings extracted from all studies, which forms the basic logical elements for subsequently constructing an overall scenario hierarchy. Each entry follows a formal relational logic, with the left

side representing perceived risk or complexity lower than the right side. As shown in the table, empirical evidence from recent literature.

#### Table 4.7 Logic Synthesis of Driving Scenario Complexity Hierarchies

| Logic Components | Synthesized Hierarchy |
| --- | --- |
| (I(l)<Rab) + (I(l) < Mrg) + (Mrg< Rab) | I(l)<Mrg < Rab |
| (H < I) + (H<I(l)) + (H < Mrg) + (I(l) < Mrg) | H<((I(l))/I)<Mrg |
| (H < (I(l)/I))<Mrg) + (I(l))<Mrg<Rab) | H<(I(l)/I)<Mrg<Rab |
| (H<(I(l)/I)<Mrg<Rab) + (Rab<I(r)) + Rab<I | H<(I(l)/I)<Mrg<Rab<(I(r)/I) |

This chapter systematically constructs a hierarchical model of risk and complexity in autonomous driving scenarios through qualitative analysis of eight primary studies.

Based on the literature evidence listed in Table 4.6, this study extracts direct comparative data on relative risk levels under different traffic environments. The study finds that although different scholars have different research focuses, they show a high degree of consistency in core conclusions such as "highways have the lowest complexity" and "intersection turning tasks have high complexity."

Through the derivation process of the logic components shown in Table 4.7, this study uses transitive inference to synthesize fragmented evidence into a complete hierarchical architecture. The final synthesized hierarchical model (Highway < Intersection (Left turn) / Intersection < Merging < Roundabout < Intersection (Right turn) / Intersection) clearly characterizes the gradient of task challenges faced by autonomous driving systems in the operating domain.

The scenario hierarchy model established in this chapter not only verifies the consistency of existing literature, but also provides theoretical support and scenario selection basis for the research on decision-making algorithms for high-complexity scenarios (such as right-turn intersections) in subsequent chapters.

# Transformer architectures in autonomous driving

## Introduction

In autonomous driving, there are various variants based on the Transformer, each designed to address specific shortcomings, such as training efficiency or suitability for different perception modalities. The following are some of the more common Transformer variants used in autonomous driving.

## Vision Transformer

#### Table 5.1: Papers that use the Vision Transformer model

Paper title

A Preference-Based Multi-Agent Federated Reinforcement Learning Algorithm Framework for Trustworthy Interactive Urban Autonomous Driving (Lu et al., 2025)

Deep Reinforcement Learning for a Self-Driving Vehicle Operating Solely on Visual Information (Lu et al., 2025)

Efficient Maximum Entropy Reinforcement Learning Framework Guided by Expert Policies: An Application in Autonomous Driving (L. Gao et al., 2025)

Improving Reinforcement Learning with Expert Demonstrations and Vision Transformers for Autonomous Vehicle Control (Elallid et al., 2024)

Integrating Driving-Aware World Model With MPC for Autonomous Driving at Unsignalized T-Intersections (X. Zhang et al., 2025)

Vision Transformer for Learning Driving Policies in Complex and Dynamic Environments (Kargar & Kyrki, 2022)

Vision Transformer (ViT) is a model that directly applies the Transformer architecture to image classification tasks. It segments images into fixed-size patches, linearly embeds these patches into a one-dimensional sequence, and then feeds it into a standard Transformer encoder for processing. ViT performs well in multiple computer vision tasks and is widely used in the field of autonomous driving for end-to-end perception and decision-making tasks.

(Audinys et al., 2025) installed a camera in front of a virtual vehicle to capture 224 × 224 pixels RGB images. In order to enable the model to infer the vehicle's current motion state (such as speed and acceleration), the input was not a single image, but rather four consecutive frames stacked together as a set of inputs.

(G. Li et al., 2025) used the Swin Transformer, a model with a hierarchical structure and a shifted window mechanism compared to ViT. This design allows the model to recognize both macroscopic objects and subtle local details in images, similar to a CNN, making it well-suited for object detection and semantic segmentation tasks. The image is divided into multiple non-overlapping local windows (e.g., 4x4 or 7x7 patches), and self-attention computation is performed only within each window.

## Decision Transformer

#### Table 5.2: Papers that use the Decision Transformer model

Paper title

Autonomous Intersection Management via Prior-Enhanced Multi-Agent Constrained Decision Transformer (R. Zhao et al., 2025)

Deep reinforcement learning navigation via decision transformer in autonomous driving (Ge et al., 2024)

Decision Transformer (DT) is a model that uses a transformer architecture to decision-making in reinforcement learning (RL). Contrary to policy gradient-based reinforcement learning, DT views the sequential decision-making problem as an autoregressive sequence modeling problem. It is a direct predictor of the future action sequence based on a sequence of states, actions and outcomes hence goal-directed decision-making.

According to (Shadab Siddiqui et al., 2025), DRLNDT has better perceptual accuracy with a success rate of up to 99.9 percent in navigation exercises. Moreover, the decision transformer architecture is able to process sequential positional data in parallel, which is much faster and more effective in training in complex environments.

## General Transformer

#### Table 5.3: Papers that use the General Transformer model

Paper title

Attention-Based Distributional Reinforcement Learning for Safe and Efficient Autonomous Driving (J. Liu et al., 2024)

Balancing Individual and Collective Interests: Integrating Transformer-Based PPO and Social Intelligence for Autonomous Vehicles at Unsignalized Intersections (Doroudian & Taghavifar, 2024)

Empirical Evaluation of Deep Learning based Autonomous Driving System with Real-Time Scene Classification Procedure (Prasath A & Navaz K, 2025)

HGRL: Human-Driving-Data Guided Reinforcement Learning for Autonomous Driving (Zhuang et al., 2024)

Improving policy training for autonomous driving through randomized ensembled double Q-learning with Transformer encoder feature evaluation (Fan et al., 2024)

Multi-Modal Attention Perception for Intelligent Vehicle Navigation Using Deep Reinforcement Learning (Z. Li et al., 2025)

Pre-trained Transformer-Enabled Strategies with Human-Guided Fine-Tuning for End-to-end Navigation of Autonomous Vehicles (Hu et al., 2024)

Safety-constrained Reinforcement Learning with Interaction-aware for Decision-making of Autonomous Driving (Zhang et al., 2025)

SoftCTRL: Soft conservative KL-control of Transformer Reinforcement Learning for Autonomous Driving (Huynh & Nguyen, 2024)

Stability-Aware Reinforcement Learning for Autonomous Driving With Dynamics-Augmented State and Lyapunov Constraints (Luo et al., 2025)

TSAC: Transformer-based Soft Actor-Critic for Behavior Decision-making in Autonomous Driving (Z. Gao et al., 2025)

Autonomous Driving Agents for Safe Road Junctions Through Deep Reinforcement Learning (Yun & Cao, 2024)

The general Transformer architecture is used as the core component of the state encoder or policy network in most studies. These are the models that are built on the typical Encoder-Decoder or encoder-only architecture but tailored to the autonomous driving tasks, e.g., by adding multimodal fusion mechanisms, e.g., CNNs and LSTMs.

These models do not only receive one form of data input, like only visual data, but instead combine data from a variety of sensors, like vehicle posture and velocity, or lidar data.

(Prasath A & Navaz K, 2025) is a combination of Transformer and CNN models. It accepts three kinds of data as input: Ego State (steering angle, velocity), Navigation Info and Surrounding Info (lidar). Since the three inputs are of varying attributes, the system applies three independent fully connected networks (FCNs) to process the ego state, navigation, and surrounding information respectively, and convert them into feature vectors of the same length (e.g., L=256).

- Other Transformers

#### Table 5.4: Papers that use the Other Transformer model

| Paper title | Transformer model |
| --- | --- |
| Augmenting Reinforcement Learning With Transformer-Based Scene Representation Learning for Decision-Making of Autonomous Driving (H. Liu et al., 2024) | multi-stage Transformer and sequential latent Transformer |
| Constraint-Guided Behavior Transformer for Centralized Coordination of Connected and Automated Vehicles at Intersections (Zhao et al., 2024) | Behavior Transformer |
| Deep Attention Driven Reinforcement Learning (DAD-RL) for Autonomous Decision-Making in Dynamic Environment (Chowdhury et al., 2024) | Spatio-Temporal Attention Encoder |
| HGRL: Human-Driving-Data Guided Reinforcement Learning for Autonomous Driving (Zhuang et al., 2024) | GPT2 |
| MTD-GPT: A Multi-Task Decision-Making GPT Model for Autonomous Driving at Unsignalized Intersections (Liu et al., 2023) | GPT2 |

Transformer architectures have also been suggested to be customized or optimized to particular autonomous driving tasks, or other pre-trained language models (including GPT-2) can be used as decision encoders.

Both (Zhuang et al., 2024) and (Liu et al., 2023) used a generative pre-training architecture that is based on GPT-2 to process decision-making in autonomous driving. MTD-GPT centers on the enhancement of the accuracy of decision-making in intersections without traffic lights using multi-task learning, whereas HGRL additionally uses human expert information and uses the causal modeling power of GPT-2 to maximize the stability and efficiency of reinforcement learning.

To improve the scene representation and accelerate the training process, (H. Liu et al., 2024) applied MST and SLT Transformers. The Behavior Transformer was applied to solve the problem of multi-vehicle conflicts and absence of rule constraints (Zhao et al., 2024). Spatio-Temporal Attention Encoder was used in (Chowdhury et al., 2024) to overcome high computational complexity and spatiotemporal interaction.

# Conclusion and outlook

This study systematically reviews academic literature from 2019 to 2025, exploring the safety performance of Deep Reinforcement Learning (DRL) algorithms based on the Transformer model in high-risk autonomous driving scenarios.

- Through data extraction and analysis from these papers, it was found that SAC performs relatively safely in most autonomous driving tasks. However, the choice of algorithm largely depends on the characteristics of the task, and there is no absolutely optimal algorithm.

- Through data extraction and logical derivation, a scenario complexity hierarchy model was derived in this study. The results show that highways < (left-turn intersections/intersections) < merging < roundabouts < (right-turn intersections/intersections). This indicates that intersection turning is currently the most challenging scenario for autonomous driving models.

- This literature review found that most studies tend to use a General Transformer as the core state encoder or policy network. This may be because the Transformer model can effectively integrate heterogeneous data such as vehicle attitude, speed, and LiDAR data through its excellent self-attention mechanism.

# Appendix A

Paper excluded due to Non Deep Reinforcement Learning

A GPT-based Decision Transformer for Multi-Vehicle Coordination at Unsignalized Intersections

A Multimodal Data Fusion and Semantic Communication Approach for Telematics

A spatial–temporal predictive transformer network for level-3 autonomous vehicle decision-making

An end-to-end autonomous driving pre-trained transformer model for multi-behavior-optimal trajectory generation

An Interpretable Image Denoising Framework via Dual Disentangled Representation Learning

Applying Neural Monte Carlo Tree Search to Unsignalized Multi-intersection Scheduling for Autonomous Vehicles

Behaviorgpt: Smart agent simulation for autonomous driving with next-patch prediction

C2F-Planner: Interaction-Aware Coarse-to-Fine Planning for Autonomous Vehicles

Deep Learning Models for Real-Time Object Recognition: Transforming Autonomous Vehicle Safety and Navigation Capabilities

DeepRide: Dashcam Video Description Dataset for Autonomous Vehicle Location-Aware Trip Description

Development and testing of an image transformer for explainable autonomous driving systems

DiffSCTG: Diffusion-based Safety-Critical Trajectory Generation for Autonomous Vehicles

DMFuser: Distilled Multi-Task Learning for End-to-end Transformer-Based Sensor Fusion in Autonomous Driving

Drivetransformer: Unified transformer for scalable end-to-end autonomous driving

Enhancing Driving Representation with Vector Quantized Encoding for Behavior Planning in Autonomous Driving

Explaining Autonomous Driving Actions with Visual Question Answering

Gameformer: Game-theoretic modeling and learning of transformer-based interactive prediction and planning for autonomous driving

Image transformer for explainable autonomous driving system

Implicit Predictive Behavior Cloning for Autonomous Driving Decision-Making in Urban Traffic

Intention-Aware Denoising Diffusion Model for Trajectory Prediction

Interaction-aware trajectory prediction for safe motion planning in autonomous driving: A transformer-transfer learning approach

Interactionnet: Joint planning and prediction for autonomous driving with transformers

Interactive Decision-Making Integrating Graph Neural Networks and Model Predictive Control for Autonomous Driving

Joint Prediction of Intention and Trajectory in Automated Driving Using Interactive Dynamic Bayesian Network

M2da: multi-modal fusion transformer incorporating driver attention for autonomous driving

Proactive Public Safety in 6G: Leveraging AI and Crowdsourcing for Critical Tasks

Social-Suitable and Safety-Sensitive Trajectory Planning for Autonomous Vehicles

Safety-enhanced autonomous driving using interpretable sensor fusion transformer

Show Me Once: A Transformer-Based Approach for an Assisted-Driving System

STdi4DMPC: Distributed Model Predictive Control for Connected and Automated Truck Platoon With Mixed Traffic Flow Based on Spatiotemporal Trajectory Prediction

Target-point Attention Transformer: A novel trajectory predict network for end-to-end autonomous driving

The Road Ahead: A Comprehensive Review of Recent Advances in Traffic Sign and Lane Line Recognition for Autonomous Systems

Trajectory Prediction Model of Electric Vehicle Autonomous Driving Based on Hybrid Attention Transformer Network

Transformer based composite network for autonomous driving trajectory prediction on multi-lane highways

Transformer Based Federated Q-Learning to Mitigate Data Poisoning Attacks for Connected Vehicles and Micromobility Devices in Blockchain Consensus

Transformer enhanced affordance learning for autonomous driving

Transformer-Based Domain Knowledge Transfer for End-to-End Autonomous Driving

Transformer-based imitative reinforcement learning for multirobot path planning

Transformer-based model for predicting trajectories in autonomous vehicle–pedestrian conflicts: a proactive approach to road safety

Transfuser: Imitation with transformer-based sensor fusion for autonomous driving

Vision transformer for detecting critical situations and extracting functional scenario for automated vehicle safety assessment

VistaGPT: Generative parallel transformers for vehicles with intelligent systems for transport automation

ViT-Enabled Task-Driven Autonomous Heuristic Navigation Based on Deep Reinforcement Learning

Paper excluded due to Non Transformers

A Hybrid RL-GNN Approach for Precise Pedestrian Trajectory Prediction in Autonomous Navigation

DQ-GAT: Towards Safe and Efficient Autonomous Driving With Deep Q-Learning and Graph Attention Networks

Evolutionary Decision-Making and Planning for Autonomous Driving: A Hybrid Augmented Intelligence Framework

Imitation Learning for Autonomous Driving Cars

Paper excluded due to No Safe Metrics

Hierarchical Transformers for Motion Forecasting Based on Inverse Reinforcement Learning

Intelligent Cooperative Sensing for Connected and Autonomous Vehicles: An Improved Decision Transformer Approach

Reinforcement Learning-Based High-Speed Path Following Control for Autonomous Vehicles

Risk-Aware Reinforcement Learning with Group Opinion for Autonomous Driving

Decision Making for Autonomous Driving Via Multimodal Transformer and Deep Reinforcement Learning

Enhancing Autonomous Driving With Spatial Memory and Attention in Reinforcement Learning

Sample-efficient Imitative Multi-token Decision Transformer for Real-world Driving

Paper excluded due to Non Specified Reinforcement Learning

A two-stage framework for parking search behavior prediction through adversarial inverse reinforcement learning and transformer

Conditional predictive behavior planning with inverse reinforcement learning for human-like autonomous driving

Empowering Autonomous Vehicles to Make Challenging Options in Unexpected Circumstances with Hybrid Learning

Enhancing Autonomous Vehicle control in complex scenarios with Deep Transformer Reinforcement Learning

Gen-Drive: Enhancing Diffusion Generative Driving Policies with Reward Modeling and Reinforcement Learning Fine-tuning

GITSR: Graph Interaction Transformer-based Scene Representation for Multi Vehicle Collaborative Decision-making

Lane Change Strategies for Autonomous Vehicles: A Deep Reinforcement Learning Approach Based on Transformer

Lightweight Strategies for Decision-Making of Autonomous Vehicles in Lane Change Scenarios Based on Deep Reinforcement Learning

Lightweight Temporal Transformer Decomposition for Federated Autonomous Driving

SPformer: A Transformer Based DRL Decision Making Method for Connected Automated Vehicles

Decision-Making for Autonomous Driving with Multiple Experts via Skill Discovery and Transformer

Optimizing Pretrained Transformers for Autonomous Driving

DTVAD: End-to-End Autonomous Vehicle Planning Using Reinforcement Learning

Safe and Interpretable Human-Like Planning With Transformer-Based Deep Inverse Reinforcement Learning for Autonomous Driving

Physics-informed Imitative Reinforcement Learning for Real-world Driving

Spatial-Temporal-Aware Safe Multi-Agent Reinforcement Learning of Connected Autonomous Vehicles in Challenging Scenarios

Safety-Aware Causal Representation for Trustworthy Offline Reinforcement Learning in Autonomous Driving

Safety-aware causal representation for trustworthy reinforcement learning in autonomous driving

Constrained Decision Transformer for Offline Safe Reinforcement Learning

Paper excluded due to Non autonomous driving

Safe Offline-to-Online Multi-Agent Decision Transformer: A Safety Conscious Sequence Modeling Approach

Summary and Reflections on Pedestrian Trajectory Prediction in the Field of Autonomous Driving

Transformer-based dual-channel self-attention for UUV autonomous collision avoidance

Transformer-based traffic-aware predictive energy management of a fuel cell electric vehicle

Dualat: Dual attention transformer for end-to-end autonomous driving

Ensembled traffic-aware transformer-based predictive energy management for electrified vehicles

D* Lite and Transformer-Enhanced SAC: A Hybrid Reinforcement Learning Framework for COLREGs-Compliant Autonomous Navigation in Dynamic Maritime Environments

ParkFormer: A Transformer-Based Parking Policy with Goal Embedding and Pedestrian-Aware Control

Path planning in dynamic structured environments using transformer-enabled twin delayed deep deterministic policy gradient for mobile robots in simulation

Paper excluded due to retracted

HiVeGPT: Human-machine-augmented intelligent vehicles with generative pre-trained transformer

Paper excluded due to can not access

Carformer: Self-driving with learned object-centric representations

Exploring Decision Transformer for Highway Automated Driving

Improving agent behaviors with rl fine-tuning for autonomous driving

Model-based reinforcement learning with self-attention mechanism for autonomous driving in dense traffic

Recent Advances in Interactive Driving of Autonomous Vehicles: Comprehensive Review of Approaches

Reinforced InterFuser for end-to-end autonomous driving in simulated environments

Training of Reinforcement Learning Agents for Autonomous Driving in Simulated Environments

Paper excluded due to secondary study/survey

A survey of autonomous driving from a deep learning perspective

A survey of vision transformers in autonomous driving: Current trends and future directions

A Survey on Recent Advancements in Autonomous Driving Using Deep Reinforcement Learning: Applications, Challenges, and Solutions

AI driven Autonomous Vehicles: Navigating the Future of Transportation

Comparison of Different Controller Architectures for Autonomous Driving and Recommendations for Robust and Safe Implementations

Deep Learning Approaches for Autonomous Driving a Comprehensive Survey

Multi-agent reinforcement learning for autonomous driving: A survey

On transforming reinforcement learning with transformers: The development trajectory

Pre-controller for Safe Reinforcement Learning using Transformer with State-Action-Reward Representations

Transformer-based models and hardware acceleration analysis in autonomous driving: A survey

Transformers in reinforcement learning: a survey

| Paper excluded | Reason/Comment |
| --- | --- |
| A Data-Driven Risk Assessment Method for Autonomous Vehicles Without Expert Rule Design | Transformer not for AD |
| A Transformer Optimized Planner for Autonomous Vehicle On-Ramping Merging Task | Offline RL |
| Learning Offline Driving Policy with Decision Transformer in Latent Space | race |
| Learning-enabled Decision-making for Autonomous Driving (Chapter 4: GameFormer) | chapter 2 no transformer, chapter no rl |
| Multimodal Vehicular Trajectory Prediction With Inverse Reinforcement Learning and Risk Aversion at Urban Unsignalized Intersections | Rare safe metric |
| DriveSafeCore: Real-Time Collision Avoidance through Spatio-Temporal Deep Learning in Autonomous Vehicles | The core objective is real-time collision avoidance. The Transformer is specifically designed for risk assessment. The system includes an Emergency Override Layer for safety oversight. |
| From Imitation to Optimization: A Comparative Study of Offline Learning for Autonomous Driving | It doesn't involve the PPO or DDPG you mentioned. Although it uses the Transformer architecture, its main purpose is to demonstrate that offline RL (CQL) is safer than pure imitation learning (BC). |
| Detrive: Imitation Learning with Transformer Detection for End-to-End Autonomous Driving | ppo not for agent |
| Toward Safe Distributed Multi-Robot Navigation Coupled With Variational Bayesian Model | not transofrmer, not driving |
| Driver Scanpath Prediction Based On Inverse Reinforcement Learning | Excluded due to mismatch in research task: focuses on human driver attention prediction rather than autonomous agent decision-making/control. |
| GRRL: Goal-Guided Risk-Inspired Reinforcement Learning for Efficient Autonomous Driving in Off- Road Environment | Unmanned Ground Vehicles |
| LC-RSS: A Lane-Change Responsibility-Sensitive Safety Framework Based on Data-Driven Lane-Change Prediction | RL was not mentioned as a core intervention or comparison object. |
| Robust Decision-Making Method for Autonomous Driving Based on Multi-Modal Fusion and Uncertainty Modeling | RL was not mentioned as a core intervention |

# Appendix B

### Turnitin Similarity Report

# Acknowledgements

# I would like to express my deepest gratitude to my supervisor, Dr. Mohammad Shadab Khan, for their invaluable guidance, patience, and continuous support throughout the duration of this manuscript. Their insights into deep reinforcement learning and autonomous driving have significantly shaped the direction of this research.

# Furthermore, I am grateful to my family and friends for their encouragement and understanding during the preparation of this manuscript. Finally, I would like to acknowledge the open-source community behind the CARLA simulator and the researchers whose primary studies provided the foundation for this literature review.

# Funding statement

There are no funding agencies supporting the research work.

Author contributions

Ng Jun Xuan: Conceptualization, Methodology, Data Curation, Formal Analysis, Investigation, Writing – Original Draft Preparation, Writing – Review & Editing.

# Conflict of interests

No conflict of interests were disclosed.

Ethics statements

Our publication ethics follow The Committee of Publication Ethics (COPE) guideline. This study is a systematic literature review and does not involve human subjects, animal experiments, or data collected from social media platforms. Therefore, no specific ethical approval was required.

# References

Audinys, R., Šlikas, Ž., Radkevičius, J., Šutas, M., & Ostreika, A. (2025). Deep reinforcement learning for a self-driving vehicle operating solely on visual information. Electronics, 14(5), 825.

Chowdhury, J., Shivaraman, V., Dangi, S., Sundaram, S., & Sujit, P. B. (2024). Deep Attention Driven Reinforcement Learning (DAD-RL) for Autonomous Decision-Making in Dynamic Environment. 2024 IEEE International Conference on Systems, Man, and Cybernetics (SMC), 3863–3868. https://doi.org/10.1109/SMC54092.2024.10832016

Doroudian, E., & Taghavifar, H. (2024). Balancing Individual and Collective Interests: Integrating Transformer-Based PPO and Social Intelligence for Autonomous Vehicles at Unsignalized Intersections. 2024 IEEE 27th International Conference on Intelligent Transportation Systems (ITSC), 3303–3308. https://doi.org/10.1109/ITSC58415.2024.10920183

Dosovitskiy, A., Ros, G., Codevilla, F., Lopez, A., & Koltun, V. (2017). CARLA: An Open Urban Driving Simulator (arXiv:1711.03938). arXiv. https://doi.org/10.48550/arXiv.1711.03938

Elallid, B. B., Benamar, N., Bagaa, M., Kelouwani, S., & Mrani, N. (2024). Improving Reinforcement Learning with Expert Demonstrations and Vision Transformers for Autonomous Vehicle Control. World Electric Vehicle Journal, 15(12), 585.

Fan, J., Zhang, X., Zou, Y., Li, Y., Liu, Y., & Sun, W. (2024). Improving policy training for autonomous driving through randomized ensembled double Q-learning with Transformer encoder feature evaluation. Applied Soft Computing, 167, 112386.

Frey, C. B., & Osborne, M. A. (2017). The future of employment: How susceptible are jobs to computerisation? Technological Forecasting and Social Change, 114, 254–280. https://doi.org/10.1016/j.techfore.2016.08.019

Fujimoto, S., van Hoof, H., & Meger, D. (2018). Addressing Function Approximation Error in Actor-Critic Methods. In J. Dy & A. Krause (Eds.), Proceedings of the 35th International Conference on Machine Learning (Vol. 80, pp. 1587–1596). PMLR. https://proceedings.mlr.press/v80/fujimoto18a.html

G. Zhao, M. Xu, Z. Wen, & J. Wang. (2025). Risk-Aware Reinforcement Learning with Group Opinion for Autonomous Driving. 2025 IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), 15254–15261. https://doi.org/10.1109/IROS60139.2025.11246043

Gao, L., Tian, H., & Wu, J. (2025). Efficient Maximum Entropy Reinforcement Learning Framework Guided by Expert Policies: An Application in Autonomous Driving. IEEE Transactions on Intelligent Transportation Systems, 26(10), 15055–15067. https://doi.org/10.1109/TITS.2025.3579033

Gao, Z., Wang, S., & Zhang, Z. (2025). TSAC: Transformer-based Soft Actor-Critic for Behavior Decision-making in Autonomous Driving. 279–283.

Ge, L., Zhou, X., Li, Y., & Wang, Y. (2024). Deep reinforcement learning navigation via decision transformer in autonomous driving. Frontiers in Neurorobotics, 18, 1338189.

Haarnoja, T., Zhou, A., Abbeel, P., & Levine, S. (2018). Soft Actor-Critic: Off-Policy Maximum Entropy Deep Reinforcement Learning with a Stochastic Actor. In J. Dy & A. Krause (Eds.), Proceedings of the 35th International Conference on Machine Learning (Vol. 80, pp. 1861–1870). PMLR. https://proceedings.mlr.press/v80/haarnoja18b.html

Hu, D., Huang, C., Wu, J., & Gao, H. (2024). Pre-trained transformer-enabled strategies with human-guided fine-tuning for end-to-end navigation of autonomous vehicles. arXiv Preprint arXiv:2402.12666.

Huynh, M. T., & Nguyen, D. D. (2024). SoftCTRL: Soft conservative KL-control of Transformer Reinforcement Learning for Autonomous Driving. arXiv Preprint arXiv:2410.22752.

Kargar, E., & Kyrki, V. (2022). Vision transformer for learning driving policies in complex and dynamic environments. 1558–1564.

Li, G., Yan, J., Qiu, Y., Li, Q., Li, J., Li, S. E., & Green, P. (2025). Lightweight Strategies for Decision-Making of Autonomous Vehicles in Lane Change Scenarios Based on Deep Reinforcement Learning. IEEE Transactions on Intelligent Transportation Systems, 26(5), 7245–7261. https://doi.org/10.1109/TITS.2025.3534797

Li, Z., Shang, T., & Xu, P. (2025). Multi-Modal Attention Perception for Intelligent Vehicle Navigation Using Deep Reinforcement Learning. IEEE Transactions on Intelligent Transportation Systems, 26(6), 8657–8669. https://doi.org/10.1109/TITS.2025.3535885

Lillicrap, T. P., Hunt, J. J., Pritzel, A., Heess, N., Erez, T., Tassa, Y., Silver, D., & Wierstra, D. (2019). Continuous control with deep reinforcement learning (arXiv:1509.02971). arXiv. https://doi.org/10.48550/arXiv.1509.02971

Liu, H., Huang, Z., Mo, X., & Lv, C. (2024). Augmenting Reinforcement Learning With Transformer-Based Scene Representation Learning for Decision-Making of Autonomous Driving. IEEE Transactions on Intelligent Vehicles, 9(3), 4405–4421. https://doi.org/10.1109/TIV.2024.3372625

Liu, J., Hang, P., Qi, X., Wang, J., & Sun, J. (2023). MTD-GPT: A Multi-Task Decision-Making GPT Model for Autonomous Driving at Unsignalized Intersections. 2023 IEEE 26th International Conference on Intelligent Transportation Systems (ITSC), 5154–5161. https://doi.org/10.1109/ITSC57777.2023.10421993

Liu, J., Yin, J., Jiang, Z., Liang, Q., & Li, H. (2024). Attention-based distributional reinforcement learning for safe and efficient autonomous driving. IEEE Robotics and Automation Letters, 9(9), 7477–7484.

Lu, S., Cai, Y., Liu, Z., Lian, Y., Chen, L., & Wang, H. (2025). A Preference-Based Multi-Agent Federated Reinforcement Learning Algorithm Framework for Trustworthy Interactive Urban Autonomous Driving. IEEE Transactions on Intelligent Transportation Systems.

Luo, Y., Liu, A., Liang, W., & Hong, J. (2025). Stability-Aware Reinforcement Learning for Autonomous Driving With Dynamics-Augmented State and Lyapunov Constraints. IEEE Robotics and Automation Letters, 10(11), 11626–11633. https://doi.org/10.1109/LRA.2025.3615032

Prasath A, Mr. A., & Navaz K, Dr. (2025). Empirical Evaluation of Deep Learning based Autonomous Driving System with Real-Time Scene Classification Procedure. 2025 4th International Conference on Innovative Mechanisms for Industry Applications (ICIMIA), 272–278. https://doi.org/10.1109/ICIMIA67127.2025.11200907

R. Zhao, Y. Fan, Y. Li, K. Wang, C. Zheng, F. Gao, & Z. Gao. (2025). Autonomous Intersection Management via Prior-Enhanced Multi-Agent Constrained Decision Transformer. IEEE Transactions on Intelligent Transportation Systems, 26(10), 15728–15745. https://doi.org/10.1109/TITS.2025.3595526

Schulman, J., Levine, S., Moritz, P., Jordan, M. I., & Abbeel, P. (2017). Trust Region Policy Optimization (arXiv:1502.05477). arXiv. https://doi.org/10.48550/arXiv.1502.05477

Schulman, J., Wolski, F., Dhariwal, P., Radford, A., & Klimov, O. (2017). Proximal Policy Optimization Algorithms (arXiv:1707.06347). arXiv. https://doi.org/10.48550/arXiv.1707.06347

Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L., Gomez, A. N., Kaiser, L., & Polosukhin, I. (2023). Attention Is All You Need (arXiv:1706.03762). arXiv. https://doi.org/10.48550/arXiv.1706.03762

X. Zhang, Z. Wu, H. Hu, J. Yang, & P. Wang. (2025). Integrating Driving-Aware World Model With MPC for Autonomous Driving at Unsignalized T-Intersections. IEEE Transactions on Intelligent Transportation Systems, 1–13. https://doi.org/10.1109/TITS.2025.3608433

Yun, J., & Cao, Z. (2024). Autonomous Driving Agents for Safe Road Junctions Through Deep Reinforcement Learning. 2024 IEEE International Conference on Agents (ICA), 39–44. https://doi.org/10.1109/ICA63002.2024.00017

Zhang, D., Luo, H., Dong, H., & Lu, J. (2025). Safety-constrained Reinforcement Learning with Interaction-aware for Decision-making of Autonomous Driving. 2025 IEEE International Conference on Multimedia and Expo (ICME), 1–6. https://doi.org/10.1109/ICME59968.2025.11209801

Zhao, R., Fan, Y., Li, Y., Wang, K., Gao, F., & Gao, Z. (2024). Constraint-guided behavior transformer for centralized coordination of connected and automated vehicles at intersections. Sensors (Basel, Switzerland), 24(16), 5187.

Zhuang, H., Chu, H., Wang, Y., Gao, B., & Chen, H. (2024). HGRL: Human-Driving-Data Guided Reinforcement Learning for Autonomous Driving. IEEE Transactions on Intelligent Vehicles, 9(12), 8089–8103. https://doi.org/10.1109/TIV.2024.3406679